Malfunction Cause & Effects

| Group | Loss of Coolant Accident |  |  |
| --- | --- | --- | --- |
| Malfunction No. | 12 |  |  |
| Description | Leakage of primary coolant into containment. Break areas up to 10% of the area of the primary tube. (A special malfunction #13 for leakages into the secondary system) |  |  |
| Option | 0: Normal Break node * 10000 + leak size in cm Primary node number 1-34, see locations in the primary loop from the nodalization scheme |  |  |
| Recommendations |  | The recommended break size is 0-500 cm, 100% corresponds to 3800 cm break area. Examples: 80100, 80200, 80500 for a cold leg break in node 8, having size 100, 200, 500 cm The recommended nodes are (examples for a 200 cm break) - loop #1 cold leg : 1 (10200) - loop #2 cold leg : 8 (80200) - loop #3 cold leg : 14 (140200) - loop #1 hot leg : 7 (70200) - loop #2 hot leg : 33 (330200) - loop #3 hot leg : 20 (200200) |  |
| Considered in Subroutine |  | SMABRE/LINK |  |
| Limitations | LOCA is physically a quite complicated event. That is why the malfunction is recommended to be used only for special training. Use Max 0.2 s for the time step for LOCA calculation. |  |  |
| Cause | A break is assumed in the primary loop as a partial rupture of the pipe itself or as a rupture of the smaller pipes connected into the primary loop. |  |  |
| Effects: - The primary coolant inventory will be reduced. - The pressurizer level drops - The primary pressure drops and the boiling in the pressurizer partly prevents the pressure decrease - The reactor is scrammed from the low pressure - The pressure continues to drop remarkably fast until the boiling starts in the core and upper plenum - The high pressure injection is activated from the low pressure signal - The pressure drops slower due to boiling between the core and steam generator and due to injections - When the primary pressure drops below the secondary pressure, heat is transferred from the secondary side to the primary side, steam is generated in the steam generator primary side too, until the tubes are empty from water. - In the beginning only water is leaking through the break, later on both water and steam - The operator may reduce the steam generator pressure during the event. The safety signal actuation may be prevented. Quite early in the transient the operator should switch off the main coolant pumps. All these actions have a significant effect on standard LOCA curve can be presented. |  |  |  |
| Group | Loss of Coolant Accident |  |  |
| Malfunction No. | 13 |  |  |
| Description | Rupture of steam generator tubes |  |  |
| Option | 0: Normal Loop# * 10000 + leak size in cm |  |  |
| Recommendations |  | The event is one variation of the SBLOCA, thus see the comments for malfunction #12. In reality the flow area of a single tube is about 2cm and a double ended break means a total flow area of 4cm for a single tube. Thus for realistic training it is recommended to use flow area of 4cm for a single tube, 8cm for two tubes, 12cm for tree etc. Thus a double ended tube break for the different loops is selected by: Loop #1 10004 Loop #2 20004 Loop #3 30004 Use Max 0.2 s for the time step. |  |
| Considered in Subroutine |  | SMABRE/LINK |  |
| Limitations |  |  |  |
| Cause | As a consequence of thermal stresses on the tube material one or several steam generator tubes are broken opening a flow connection between primary and secondary loops. |  |  |
| Effects: - On the primary side the event behaves like a small break LOCA until the primary pressure drops down to the level of the secondary pressure. After that the flow in the break is controlled by small pressure differences between the primary and the secondary side. - In the broken steam generator the secondary water level rises filling the secondary side gradually. - Operator actions are expected for the isolation of the broken steam generator and for reducing the primary injection . The selected operator actions are very important for how the event continues. |  |  |  |
| Group | Main Steam System |  |
| Malfunction No. | 52 |  |
| Description | Main steam line break, nonisolable (inside or outside containment) |  |
| Option | 0: Normal 110000: Loop #1 (inside) 120000: Loop #2 (inside) 130000: Loop #3 (inside) 210000: Loop #1 (outside) 220000: Loop #2 (outside) 230000: Loop #3 (outside) add leak size in cm Example : A 1000cm main steam line rupture for the different loops is selected by : Loop #1 inside containment : 111000 Loop #2 inside containment : 121000 Loop #3 inside containment : 131000 Loop #1 outside containment : 211000 Loop #2 outside containment : 221000 Loop #3 outside containment : 231000 |  |
| Recommendations |  |  |
| Considered in Subroutine |  | SMABRE/LINK |
| Limitations | The event is a variation of a LOCA. See comments for malfunction #12. Max time step 0.2 s. |  |
| Cause | The whole main steam line pipe or a part of it is assumed broken. Because the secondary isolation valves are located in the turbine hall the possible combinations are - nonisolable leak inside containment - nonisolable leak outside containment |  |
| Effects: * Small leakages - For small leak sizes (<100 cm ) the transient is quite mild showing only an unbalance between the feedwater injection and steam flow to the turbines. The unbalance may be seen in the steam flow measurement for the broken loop, too In a longer time interval the smaller leak empties the feedwater inventory in the condenser and this cause turbine trip. The rest of the consequences are corresponding to a larger steam line break. * Large leakage ( Max 2000 cm ) - The secondary pressure drops - The steam flow to the turbines is reduced due to lowered secondary pressure - The primary side is cooling down dropping the pressurizer level and the primary pressure - The reactor control tries to increase the core power but the upper limitation prevents the rise from getting too high - If the leak is located inside the containment, containment pressure is increased starting the containment isolation signal - In case of containment isolation signal the reactor is tripped causing turbine trip - In case of leak outside containment the turbine is tripped on low secondary pressure causing reactor trip (?) - The secondary pressure continues to drop causing activation of S/G isolation signal. All S/G-s are isolated by fast acting valves. - After isolation the intact S/G-s remain on pressure level, a slight pressure increase is caused by the heat form the primary side - The broken S/G cools the primary loop where the HPIS may be started from low pressure. This may gradually cause an overpressure. - The decay heat from the core gradually empties the broken S/G - When the broken S/G is empty the primary loop starts to heat up again. The heat-up may be so large that the spray is needed in order to limit the pressure rise - If the operator has stopped the RCP’s the cold leg of the broken S/G cools down much below the temperature in the intact loops. |  |  |